| Subdivision (Stage) / Worksite: |  |
| --- | --- |
| Date of Inspection : |  |
| Location (Pit Number to Pit Number): |  |

| Category | Checklist Items | Reference | Yes | No | N/A | WCC – HP (add initial next to box) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Safety & Environment | All Employees Inducted and signed onto SWMS All environmental controls implemented as per the approved EMP Trench Depths – Any trenches over 1.5m deep must be battered or shored and notification given to WorkSafe. |  | □ | □ |  |  |  |
| Set Out Hold Point | Has Vertical/Horizontal alignment been checked and approved? |  | □ | □ | □ | □ |  |
| Pipes / Culverts | Is bedding of sufficient depth & compacted? |  | □ | □ | □ |  |  |
|  | Are any pipes sitting on collars causing ‘beaming’? |  |  |  |  |  |  |
|  | Is the direction of laying satisfactory? (i.e. collar on upstream end?) |  | □ | □ | □ |  |  |
|  | Is rubber ring jointing satisfactory? |  | □ | □ | □ |  |  |
|  | If flush jointed pipes have been accepted, rubber bands should be used at the joints. Are they in place and in sound condition? | IDM 16.10.1.1 | □ | □ | □ |  |  |
|  | Are all pipes sound and not cracked? |  | □ | □ | □ |  |  |
|  | Have all house drains been connected to top of pipe with an acceptable connection? | IDM 16.16 | □ | □ | □ |  |  |
|  | Are lifting-hole bungs in place? |  | □ | □ | □ |  |  |
|  | Do the size of pipes, bedding materials and compaction conform to design? Compaction Tests & Material Tests to be submitted to WCC. | IDM 16.10.3 | □ | □ | □ |  |  |
| In-situ Pits Hold Point | Are inner and outer formworks and reinforcement in place? |  | □ | □ | □ | □ |  |
| In-situ Pits | Has the base material been placed and compacted as required? |  | □ | □ | □ |  |  |
|  | Are all collars outside the pits walls? |  | □ | □ | □ |  |  |
|  | Does the strength of concrete used to construct pits meet the requirements of the Manual? (min. 25MPa) | Refer relevant IDM drawing in 400 Series | □ | □ | □ |  |  |
| Pre-cast Pits | Have all precast pits have been supplied and installed as per plan? |  | □ | □ | □ |  |  |
|  | Have all holes been mortared up for precast pits? |  | □ | □ | □ |  |  |
| As-Cons | Have all pipe Invert Levels and pits been surveyed for as-constructed drawings? Survey pickup for as-constructed drawings in D-Spec format. | IDM 7.6 A-Spec Guidelines | □ | □ | □ |  |  |
| CCTV | Are all underground drainage lines clear of debris and sediment? CCTV verification to be submitted to WCC with QA Package. | IDM 7.9 | □ | □ | □ |  |  |
| Consent to Proceed to Next Stage? |  |  | □ | □ |  | □ |  |

Additional Comments:

| Contractor/Superintendent Name: | Signature: | Date: |
| --- | --- | --- |
| WCC Construction Supervisor Name: | Signature: | Date: |