Detailed Site Plan 																	Project:

Date and Revision:

Include types and locations of Environment Protection Measures

Insert plan here

SITE ENVIRONMENTAL PROTECTION MEASURES

| Site Environmental Protection Measures |  | 5. Erosion and Sediment Risk: Significant/Medium/Low |  |
| --- | --- | --- | --- |
| The following aspects shall be managed with the environmental protection measures outlined in this plan |  | Objective: Limit erosion from disturbed or unstable soil and prevent sediment from entering stormwater drains and waterways. |  |
|  |  | Drainage Management | Stockpile protection |
|  |  | Any contaminated flow generated on site to be diverted to a sediment trap or settlement treatment prior to release from the site or into receiving waters. Site runoff to be diverted away from stockpiles. | Minimise the number and size of stockpiles – maximum 2:1 height to width ratio. Stockpiles to be placed at least 10m from drainage inlets, open drains, water course and paved areas. A cut off drain with earth bund to be installed on the upslope side of the stockpile to divert runoff away from the stockpile. Sediment retention structures to be place downslope of all stockpiles. Stockpiles in place for more than 28 days must be temporarily grassed |
| - Management |  |  |  |
| Responsibilities |  |  |  |
| Include the names and contact details of those responsible for the implementation of the EMP and works on site (eg contractors, site supervisors, developers) Emergency Contact 1: Emergency Contact 2: |  |  |  |
|  |  | Soil stabilisation |  |
| Communication of EMP requirements | Informing residents | Sediment traps |  |
| All contractors, subcontractors and others working on site to be inducted into SEMP. The SEMP is to be displayed in visible locations within site compound/office Other forms of communications such as toolbox meetings | State when and how this will occur (if necessary) including which residents will be informed | Sediment runoff controls and drainage around all construction areas must be established prior to the commencement of any works. All sediment control measures must be maintained and intact for the duration of the works and inspected regularly including prior to and post rainfall events to ensure they are functioning properly. Immediate repair of damaged sediment control measures after any flood event to be undertaken. Extra sediment fencing and other sediment control measures are to be stockpiled on site for emergency repairs. Sediment fencing (or other acceptable sediment control measures) to be installed downslope of disturbed areas. Stormwater pits along established roadway, which are subject to sediment, must be protected. | Vehicle and Road Management |
|  |  |  | Site access Cleaning vehicles Wash all equipment in designated wash down area. Sediment fencing (or other acceptable sediment control measure) to be erected around vehicle washdown area. Street cleaning Existing and finished roads to be inspected regularly and any sediments deposited there will be fully removed until the end of the maintenance period. Roads are to be cleared prior to rainfall/storm events. |
|  | Associated documents |  |  |
| Inspections and Maintenance |  |  |  |
| EMP protection measures to be monitored at least weekly, as well as before and after rain events, with immediate repairs made to sediment controls. |  |  |  |
| Staging of works |  |  |  |
| Details if applicable |  |  |  |
| - Noise and Vibration Risk: Significant/Medium/Low |  |  |  |
| Objective: Minimise noise and vibration at all times. EPA Victoria and Council requirements must be adhered to in relation to the level of noise and working hours. |  |  |  |
|  |  | Dewatering |  |
|  |  | If there are no suitable vegetated areas, turbidity exceeds 30 NTU and is higher than upstream measurements, treatment of sediment laden water will be undertaken prior to discharging runoff to a natural waterway or stormwater system. Hourly measurements of discharge water quality will be taken. |  |
| Working hours | Noise Minimisation Methods |  |  |
| am to pm Mon - Fri am to pm Sat |  |  | Other |
| Vibration Minimisation Methods |  | 6. Waste Risk: Significant/Medium/Low |  |
|  |  | Objective: Prevent waste from polluting the environment and dispose of waste in a responsible manner. |  |
| - Dust Risk: Significance/Medium/Low |  | Movement of Soil: Offsite/ On Site/ NA | Waste Minimisation Methods |
| Objective: Dust generation and transport of dust must be minimised. |  | Contaminant Status: | Separate recyclable waste and materials from general waste for recycling Recycling bins clearly marked to avoid contamination of recyclable materials |
| Minimising Dust Generation | Contingencies |  |  |
| Any activity involving the handling and moving of soil to be restricted on dry windy days. | Stop work if dust generated from site reaches neighbouring areas or properties, if visibility is affected on adjoining roads or if dust on the work site is a risk to occupational health. | Waste Storage and Disposal | Concrete Washout areas |
|  |  | Bins to have lids to prevent rubbish from blowing away. Lidded rubbish and recyclables bins to be provided close to site office and/or lunch eating area for non-construction waste generated on site. Bins to be emptied before becoming full. Worksites are to be free of litter – any visible litter to be collected daily. Contain all waste material on site in accordance with regulatory requirements. Adhere to regulatory requirement for waste disposal |  |
| Dust Suppression |  |  |  |
| All loads of soil taken off site for disposal must be covered. |  |  |  |
|  | Other |  |  |
|  |  |  | Other |
| - Chemicals Risk: Significant/Medium/Low |  |  |  |
| Objective: Prevent spills and leaks of chemicals from onsite storage and handling practices. |  | 7. Significant Flora/Fauna Risk: Significant/Medium/Low |  |
|  |  | Requirement: All significant flora on and adjacent to the site must be protected |  |
| Storage | Refuelling Procedure | Yes/No. Details: |  |
|  | No refuelling to occur within 30 mtres of any waterway or within 10m of any drainage system that discharges to a waterway |  |  |
| Spill Management |  | 8. Weeds and Pest Animals |  |
|  |  | Objective: Weeds and pest animals must not adversely impact native vegetation or waterways. |  |
| Other |  |  |  |
| Other Considerations |  | 9. Archaeological/Heritage Risk: Significant/Medium/Low |  |
|  |  | Requirement: Places, sites and objects of archaeological or heritage significance must be protected. |  |
|  |  | Yes/No. Details: |  |

| RISK ASSESSMENT CHECKLIST |  | Site EMP A1 Plan (2)- Risk Assessment and Designs of Environmental Protection Measures Project Name: ##### Date and Revision: |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Noise |  |  |  |  |  |
| Issues: - Nature of Noise Generating Works: - Potential Noise Receptors: - Proximity of Works to Noise Receptors: | Likelihood |  |  |  |  |
|  | Consequence |  |  |  |  |
|  |  | ##### Environmental protection measures shall be constructed in accordance with the following designs. ##### DESIGNS HERE |  |  |  |
|  | Overall Risk |  |  |  |  |
| - Dust |  |  |  |  |  |
| Issues: - Dust Sources: - Potential Dust Receptors: - Proximity of Works to Dust Receptors: - Extent of Exposed Earth and Duration of Time Exposed: - Wind Conditions: | Likelihood |  |  |  |  |
|  | Consequence |  |  |  |  |
|  | Overall Risk |  |  |  |  |
| - Erosion and Sediment |  |  |  |  |  |
| Issues: - Erosion and Sediment Sources: - Potential Erosion and Sediment Receptors: - Proximity of Works to Erosion and Sediment Receptors: - Extent of Exposed Earth and Duration of Time Exposed: - Soil Type and Erosivity: - Slope: - Site Drainage Regime: - Rainfall: - Vehicle Movements On and Off Site: | Likelihood |  |  |  |  |
|  | Consequence |  |  |  |  |
|  | Overall Risk |  |  |  |  |
| - Waste |  |  |  |  |  |
| Issues: - Nature of Waste to be Generated: - Presence of Waste On Site Prior to Work Commencement: - Quantity of Waste Anticipated: - Potential Waste Receptors: - Proximity to Potential Waste Receptors: | Likelihood |  |  |  |  |
|  | Consequence |  |  |  |  |
|  | Overall Risk |  |  |  |  |
| - Chemicals |  |  |  |  |  |
| Issues: - Types of Chemicals and Fuels Used and/or Stored On Site: - Quantities of Chemicals and Fuels Used and/or Stored On Site: - Potential Chemical Receptors: - Proximity to Potential Chemical Receptors: | Likelihood |  |  |  |  |
|  | Consequence |  |  |  |  |
|  | Overall Risk |  |  |  |  |
| - Significant Flora/ Fauna |  |  |  |  |  |
| Issues: - Types of Flora/ Fauna: - Vulnerability of Flora/ Fauna: - Proximity of Flora/Fauna to Works: - Work Activities Which May Threaten Flora/ Fauna: - Potential Impacts on Flora/ Fauna: | Likelihood |  |  |  |  |
|  | Consequence |  |  |  |  |
|  | Overall Risk |  |  |  |  |
| - Archaeological/ Heritage |  |  |  |  |  |
| Issues: - Traditional Land Owners Consulted? Yes/ No - Survey or Assessment Conducted? Yes/ No/ Not Required - Probability of Encountering Archaeological/ Heritage Items During Works: - Types of Archaeological/ Heritage Items On Site: - Proximity of Archaeological/ Heritage Items to Works On Site: - Work Activities Which May Threaten Archaeological/ Heritage Items: - Potential Impacts on Archaeological/ Heritage Items: | Likelihood |  |  |  |  |
|  |  | #####  |  | #####  |  |
|  | Consequence | Issues: | ##### Likelihood | Issues: | ##### Likelihood |
|  |  |  | ##### Consequence |  | ##### Consequence |
|  | Overall Risk |  |  |  |  |
|  |  |  | ##### Overall Risk |  | ##### Overall Risk |

I have read this Environmental Management Plan and agree to undertake works and ensure sub-contractors undertake works in accordance with this plan.

Developer _________________		 Consultant_________________ 	Contractor_________________